Homework 9

Computer Architecture

- A digital computer has a common bus system for 16 registers of 32 bits each. The bus is constructed with multiplexers.

  - How many selection inputs are there in each multiplexer?

  - What size of multiplexers are needed?

  - How many multiplexers are there in the bus?

- The adder-subtractor circuit has the following values for input mode M and data inputs A and B. In each case, determine the values of the output: S3, S2, S1, S0, and C4.

|  | M | A | B |
| --- | --- | --- | --- |
| a. | 0 | 0111 | 0110 |
| b. | 1 | 1100 | 1000 |

- Design an arithmetic circuit with one selection variable S and two n-bit data inputs A and B. The circuit generates the following four arithmetic operations in conjunction with the input carry Cin. Draw the logic diagram for the first two stages.

| S | Cin = 0 | Cin = 1 |
| --- | --- | --- |
| 0 | D = A + B (add) | D = A + 1 (increment) |
| 1 | D = A – 1 (decrement) | D = A + B' + 1 (subtract) |